VIVEKANANDHA COLLEGE OF ENGINEERING FOR WOMEN

[Autonomous]

An ISO 9001:2015 Certified Institution

(Approved by AICTE, New Delhi and Affiliated to Anna University, Chennai,

Accredited by NAAC and NBA Tier - I UG: CSE, EEE, ECE, IT and Bio-Tech.)

Tiruchengode - 637205, Namakkal Dt., Tamil Nadu

AI-ENABLED PREDICTIVE MONITORING AND ENERGY OPTIMIZATION OF RENEWABLE MICROGRIDS USING IoT

A MINI PROJECT REPORT

Submitted by

[TO BE PROVIDED]

[TO BE PROVIDED]

In partial fulfillment for the award of the degree of

[TO BE PROVIDED]

IN

[TO BE PROVIDED]

VIVEKANANDHA COLLEGE OF ENGINEERING FOR WOMEN

[Autonomous]

An ISO 9001:2015 Certified Institution

(Approved by AICTE, New Delhi and Affiliated to Anna University, Chennai,

Accredited by NAAC and NBA Tier - I UG: CSE, EEE, ECE, IT and Bio-Tech.)

Tiruchengode - 637205, Namakkal Dt., Tamil Nadu

[TO BE PROVIDED]

VIVEKANANDHA COLLEGE OF

ENGINEERING FOR WOMEN

BONAFIDE CERTIFICATE

Certified that this project report “AI-ENABLED PREDICTIVE MONITORING AND ENERGY OPTIMIZATION OF RENEWABLE MICROGRIDS USING IoT” is the bonafide work of “[TO BE PROVIDED]” who carried out the mini project work under my supervision.

Submitted to the Viva voce Examination held on _____________________.

| ____________________________ Signature of the Head of the Department Name: [TO BE PROVIDED] Department: [TO BE PROVIDED] Address: [TO BE PROVIDED] | ____________________________ Signature of the Supervisor Name: [TO BE PROVIDED] Designation: [TO BE PROVIDED] Department: [TO BE PROVIDED] |
| --- | --- |

| INTERNAL EXAMINER | EXTERNAL EXAMINER |
| --- | --- |

# ACKNOWLEDGEMENT

We express our sincere gratitude to our project supervisor, [TO BE PROVIDED], [TO BE PROVIDED], Department of [TO BE PROVIDED], for the valuable guidance, constant encouragement and constructive suggestions provided throughout the course of this mini project.

We thank the Head of the Department, [TO BE PROVIDED], and the entire faculty and non-teaching staff of the Department of [TO BE PROVIDED] for their support and for providing the laboratory facilities that made this work possible.

We are grateful to the Principal and the Management of Vivekanandha College of Engineering for Women for the infrastructure and encouragement extended to us.

Finally, we thank our family and friends for their patience and moral support during the preparation of this report.

THE CANDIDATES

# INSTITUTE AND DEPARTMENT VISION AND MISSION

### Institute Vision

[TO BE PROVIDED]

### Institute Mission

[TO BE PROVIDED]

### Department Vision

[TO BE PROVIDED]

### Department Mission

[TO BE PROVIDED]

# PROGRAMME EDUCATIONAL OBJECTIVES, PROGRAMME OUTCOMES AND PROGRAMME SPECIFIC OUTCOMES

### Programme Educational Objectives

PEO 1: [TO BE PROVIDED]

PEO 2: [TO BE PROVIDED]

PEO 3: [TO BE PROVIDED]

PEO 4: [TO BE PROVIDED]

PEO 5: [TO BE PROVIDED]

### Programme Outcomes

PO1: Engineering knowledge - Apply the knowledge of mathematics, science, engineering fundamentals and an engineering specialization to the solution of complex engineering problems.

PO2: Problem analysis - Identify, formulate, review research literature and analyze complex engineering problems reaching substantiated conclusions using first principles of mathematics, natural sciences and engineering sciences.

PO3: Design/development of solutions - Design solutions for complex engineering problems and design system components or processes that meet the specified needs with appropriate consideration for public health and safety and the cultural, societal and environmental considerations.

PO4: Conduct investigations of complex problems - Use research-based knowledge and research methods including design of experiments, analysis and interpretation of data, and synthesis of the information to provide valid conclusions.

PO5: Modern tool usage - Create, select and apply appropriate techniques, resources and modern engineering and IT tools including prediction and modelling to complex engineering activities with an understanding of the limitations.

PO6: The engineer and society - Apply reasoning informed by the contextual knowledge to assess societal, health, safety, legal and cultural issues and the consequent responsibilities relevant to the professional engineering practice.

PO7: Environment and sustainability - Understand the impact of the professional engineering solutions in societal and environmental contexts, and demonstrate the knowledge of, and the need for, sustainable development.

PO8: Ethics - Apply ethical principles and commit to professional ethics and responsibilities and norms of the engineering practice.

PO9: Individual and team work - Function effectively as an individual, and as a member or leader in diverse teams, and in multidisciplinary settings.

PO10: Communication - Communicate effectively on complex engineering activities with the engineering community and with society at large.

PO11: Project management and finance - Demonstrate knowledge and understanding of the engineering and management principles and apply these to one's own work, as a member and leader in a team, to manage projects and in multidisciplinary environments.

### Programme Specific Outcomes

PSO1: [TO BE PROVIDED]

PSO2: [TO BE PROVIDED]

PSO3: [TO BE PROVIDED]

# COURSE OUTCOME MAPPING WITH PROGRAMME OUTCOMES AND PROGRAMME SPECIFIC OUTCOMES

Upon completion of the course, students will be able to:

Course outcomes and their Bloom's taxonomy level

| CO# | Course Outcomes | Bloom's Taxonomy Level |
| --- | --- | --- |
| CO1 | Identify the area of the project | Remembering |
| CO2 | Illustrate the literature of the project | Understanding |
| CO3 | Determine the problem identified and plan the solution | Applying |
| CO4 | Experimentation / design analysis / prototype development | Creating |
| CO5 | Result and analysis | Analyzing |
| CO6 | Conclusion, scope for future work and documentation | Evaluating |

### CO - PO / PSO Mapping

The following mapping is proposed for this mini project. The values 3, 2 and 1 indicate a high, medium and low correlation respectively and 0 indicates no correlation. The mapping should be confirmed with the project supervisor.

CO - PO / PSO mapping matrix (3/2/1 = strong/medium/low correlation)

| COs | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO7 | PO8 | PO9 | PO10 | PO11 | PSO1 | PSO2 | PSO3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO1 | 3 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 0 |
| CO2 | 2 | 3 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| CO3 | 2 | 2 | 3 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 1 | 0 |
| CO4 | 2 | 1 | 3 | 3 | 2 | 1 | 0 | 0 | 2 | 1 | 0 | 2 | 2 | 1 |
| CO5 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 2 |
| CO6 | 2 | 1 | 1 | 1 | 0 | 2 | 2 | 1 | 0 | 0 | 2 | 1 | 1 | 2 |

# RELEVANCE TO PROGRAMME OUTCOMES AND PROGRAMME SPECIFIC OUTCOMES

This mini project applies engineering knowledge to a real sustainability problem and therefore maps strongly to PO1 and PO2. The behaviour of a hybrid solar and wind microgrid was studied, the monitoring and prediction problem was formulated, and the measured and simulated sensor data were analysed using first principles.

The design and development of the embedded node, the data pipeline and the machine learning models address PO3 and PO4. The work included the design of the sensor interface, the construction of a reproducible dataset and the quantitative evaluation of the models on a held-out test set.

The use of the ESP8266 platform, the Flask framework, SQLite and the scikit-learn library satisfies PO5, since modern engineering and IT tools were selected and applied with an understanding of their limitations.

The project contributes to affordable and clean energy and to climate action, which aligns with PO6 and PO7. The report also documents the ethical treatment of data by never presenting simulated or estimated values as measured values, which supports PO8.

The work was carried out as a team, with clearly divided responsibilities for the firmware, the backend, the machine learning pipeline and the dashboard, which corresponds to PO9 and PO10. The planning of the schedule and the documentation of the deliverables relate to PO11.

The project also supports the Sustainable Development Goals, particularly SDG 7 (Affordable and Clean Energy), SDG 9 (Industry, Innovation and Infrastructure) and SDG 13 (Climate Action).

# ABSTRACT

Renewable microgrids that combine solar photovoltaic and small wind generation are widely used, but their output is intermittent and developing faults often go unnoticed. This mini project designs and evaluates a low-cost Internet of Things (IoT) platform that monitors a hybrid solar and wind microgrid node, predicts its generation, classifies its operating condition, detects anomalies, and recommends how the available energy should be allocated between essential load, deferrable load and storage. An ESP8266 node measures solar and wind voltage, turbine speed, temperature, humidity and light level and publishes each reading over Wi-Fi to a Flask REST backend, which validates, deduplicates and stores the data and runs three Random Forest models. On a reproducible simulated dataset of 12,000 samples with a chronological split, the generation regressor reached a mean absolute error of 0.0147 W and an R-squared of 0.9968, the condition classifier an accuracy of 0.8528, and the anomaly detector an accuracy of 0.9861. These results characterise the software pipeline on simulated data and are not a claim of field accuracy.

Keywords: IoT, renewable microgrid, solar and wind energy, Random Forest, generation forecasting, anomaly detection, energy management, ESP8266, Flask, scikit-learn.

TABLE OF CONTENTS

ACKNOWLEDGEMENT	ii

INSTITUTE AND DEPARTMENT VISION AND MISSION	iii

Institute Vision	iii

Institute Mission	iii

Department Vision	iii

Department Mission	iii

PROGRAMME EDUCATIONAL OBJECTIVES, PROGRAMME OUTCOMES AND PROGRAMME SPECIFIC OUTCOMES	iv

Programme Educational Objectives	iv

Programme Outcomes	iv

Programme Specific Outcomes	v

COURSE OUTCOME MAPPING WITH PROGRAMME OUTCOMES AND PROGRAMME SPECIFIC OUTCOMES	vii

CO - PO / PSO Mapping	vii

RELEVANCE TO PROGRAMME OUTCOMES AND PROGRAMME SPECIFIC OUTCOMES	viii

ABSTRACT	x

LIST OF TABLES	xiv

LIST OF FIGURES	xv

LIST OF SYMBOLS AND ABBREVIATIONS	xvi

INTRODUCTION	1

1.1 OVERVIEW OF RENEWABLE MICROGRIDS	1

1.2 NEED FOR PREDICTIVE MONITORING IN MICROGRIDS	1

1.3 ROLE OF THE INTERNET OF THINGS	2

1.4 MACHINE LEARNING FOR ENERGY ANALYTICS	2

1.5 PROBLEM STATEMENT	2

1.6 OBJECTIVES	3

1.7 SCOPE OF THE PROJECT	4

1.8 ORGANISATION OF THE REPORT	4

LITERATURE REVIEW	5

2.1 INTRODUCTION	5

2.2 IOT ARCHITECTURES FOR ENERGY MONITORING	5

2.3 SOLAR AND WIND GENERATION MODELLING	6

2.4 MACHINE LEARNING FOR GENERATION PREDICTION	6

2.5 ANOMALY AND FAULT DETECTION	7

2.6 ENERGY MANAGEMENT AND LOAD ALLOCATION	7

2.7 RESEARCH GAP	7

2.8 SUMMARY	8

SYSTEM DESIGN AND METHODOLOGY	9

3.1 INTRODUCTION	9

3.2 SYSTEM ARCHITECTURE	9

3.3 HARDWARE DESIGN	10

3.4 SENSOR NODE FIRMWARE	11

3.5 DATA CONTRACT AND REST API	11

3.6 DATABASE DESIGN	12

3.7 DATA PIPELINE AND FEATURE ENGINEERING	13

3.8 DATASET GENERATION	15

3.9 MACHINE LEARNING MODELS	15

3.10 ENERGY ALLOCATION POLICY	16

3.11 DASHBOARD	16

3.12 SOFTWARE AND TOOLCHAIN	17

3.13 TESTING APPROACH	17

3.14 SUMMARY	17

RESULTS AND DISCUSSION	19

4.1 INTRODUCTION	19

4.2 DATASET CHARACTERISTICS	19

4.3 SENSOR AND GENERATION ANALYSIS	20

4.4 GENERATION PREDICTION RESULTS	22

4.5 OPERATING-CONDITION CLASSIFICATION RESULTS	24

4.6 ANOMALY DETECTION RESULTS	25

4.7 FEATURE IMPORTANCE	26

4.8 ENERGY OPTIMIZATION RESULTS	27

4.9 API AND DASHBOARD VALIDATION	28

4.10 LIMITATIONS OF THE EVALUATION	28

4.11 SUMMARY	29

CONCLUSION AND FUTURE SCOPE	30

5.1 CONCLUSION	30

5.2 FUTURE SCOPE	31

5.3 SUSTAINABLE DEVELOPMENT GOALS	32

APPENDIX 1 - PIN CONFIGURATION AND WIRING	33

APPENDIX 2 - SAMPLE API EXCHANGE	34

APPENDIX 3 - INSTALLATION AND OPERATING GUIDE	35

REFERENCES	36

# LIST OF TABLES

| Table No. | Title | Page No. |
| --- | --- | --- |
| 2.1 | Summary of related work and the research gap | 8 |
| 3.1 | Hardware components and their roles | 10 |
| 3.2 | ESP8266 pin map | 11 |
| 3.3 | Ingest payload fields | 12 |
| 3.4 | REST API endpoints | 12 |
| 3.5 | Database tables and their purpose | 13 |
| 3.6 | Features used by the machine learning models | 14 |
| 3.7 | Model hyperparameters | 15 |
| 3.8 | Energy-allocation tiers and default values | 16 |
| 3.9 | Software, libraries and their roles | 17 |
| 4.1 | Dataset composition | 19 |
| 4.2 | Anomaly-type distribution | 20 |
| 4.3 | Generation regressor performance | 23 |
| 4.4 | Operating-condition classification report (held-out test set) | 24 |
| 4.5 | Anomaly detector performance on the held-out test set | 25 |
| 4.6 | Feature importance of the generation regressor | 27 |
| 4.7 | Energy-allocation scenarios | 27 |

# LIST OF FIGURES

| Figure No. | Title | Page No. |
| --- | --- | --- |
| 3.1 | System architecture of the IoT microgrid monitoring platform | 10 |
| 3.2 | Entity-relationship diagram of the SQLite database | 13 |
| 3.3 | Online and offline data pipelines sharing one feature-engineering implementation | 14 |
| 4.1 | Distribution of operating conditions in the dataset | 20 |
| 4.2 | Solar voltage and solar power over time | 21 |
| 4.3 | Wind voltage and turbine RPM over time | 21 |
| 4.4 | Light level versus solar generation | 21 |
| 4.5 | Turbine RPM versus wind generation | 22 |
| 4.6 | Relative contribution of solar and wind generation | 22 |
| 4.7 | Actual versus predicted power on the held-out test set | 23 |
| 4.8 | Regression evaluation and residuals | 23 |
| 4.9 | Generation trend and forecast | 23 |
| 4.10 | Comparison of model scores | 24 |
| 4.11 | Confusion matrix for the operating-condition classifier | 25 |
| 4.12 | Confusion matrix for the anomaly detector | 26 |
| 4.13 | Normal versus anomalous sensor patterns | 26 |
| 4.14 | Feature importance of the generation regressor | 27 |

# LIST OF SYMBOLS AND ABBREVIATIONS

| AC | Alternating Current |
| --- | --- |
| ADC | Analogue-to-Digital Converter |
| AI | Artificial Intelligence |
| API | Application Programming Interface |
| CSV | Comma-Separated Values |
| DC | Direct Current |
| DOI | Digital Object Identifier |
| ER | Entity-Relationship |
| GPIO | General-Purpose Input/Output |
| HTML | HyperText Markup Language |
| HTTP | HyperText Transfer Protocol |
| I2C | Inter-Integrated Circuit |
| IoT | Internet of Things |
| INA219 | Current, Voltage and Power Monitor |
| JSON | JavaScript Object Notation |
| LDR | Light-Dependent Resistor |
| MAE | Mean Absolute Error |
| ML | Machine Learning |
| MPP | Maximum Power Point |
| NTP | Network Time Protocol |
| PEO | Programme Educational Objective |
| PO | Programme Outcome |
| PSO | Programme Specific Outcome |
| RF | Random Forest |
| RMSE | Root Mean Square Error |
| RPM | Revolutions Per Minute |
| REST | Representational State Transfer |
| SDG | Sustainable Development Goal |
| SQL | Structured Query Language |
| SSR | Solid-State Relay |
| TOC | Table of Contents |
| VOC | Open-Circuit Voltage |
| WSN | Wireless Sensor Network |

CHAPTER 1

# INTRODUCTION

## 1.1 OVERVIEW OF RENEWABLE MICROGRIDS

A microgrid is a small, self-contained electrical system that combines distributed generation sources with local loads and, optionally, energy storage. When the generators are renewable, the microgrid can supply electricity with very low carbon emissions and at a low marginal cost. Hybrid systems that combine solar photovoltaic generation with small wind generation are attractive because the two sources are complementary: solar output peaks during the day while wind output can continue into the night. Such systems are widely used for rural electrification, agricultural pumping, telecom towers and campus demonstration facilities.

## 1.2 NEED FOR PREDICTIVE MONITORING IN MICROGRIDS

Renewable generation is intermittent and depends on weather that cannot be controlled. Because the available power changes from minute to minute, operators need continuous information about how much energy is being produced, whether the equipment is healthy and how the available energy should be used. Faults such as a shaded or soiled panel, a fouled turbine, a failed rectifier or a drifting sensor may not stop the system immediately, but they silently reduce the energy delivered and can shorten equipment life. If such conditions are detected early, maintenance can be scheduled before a minor degradation becomes a failure.

## 1.3 ROLE OF THE INTERNET OF THINGS

The Internet of Things provides the sensing, communication and actuation layer that makes continuous monitoring practical. Low-cost microcontrollers such as the ESP8266 integrate Wi-Fi connectivity and analogue and digital inputs in a single module, so a complete telemetry node can be built at very low cost. The node samples the sensors, constructs a compact message and publishes it to a server over the local wireless network, where it can be stored, analysed and displayed.

## 1.4 MACHINE LEARNING FOR ENERGY ANALYTICS

Once a reliable stream of sensor data is available, machine learning can add value in three ways. First, a regression model can estimate the generation that the system should be producing and provide a short-horizon forecast. Second, a classification model can label the current operating condition, for example normal, reduced generation, night idle or fault. Third, an anomaly-detection model can flag readings that do not fit the expected pattern, which usually indicates a sensor fault or an equipment problem. Random Forest models are well suited to this task because they handle mixed-scale sensor features, are robust to noise and provide feature importances that can be explained to an operator.

## 1.5 PROBLEM STATEMENT

Small renewable microgrids are often installed without any monitoring, so their actual performance is unknown and developing faults go unnoticed until the output collapses. Commercial monitoring systems are expensive and are usually closed, which makes them unsuitable for low-cost installations and for student projects. There is a need for an open, low-cost platform that continuously monitors a hybrid solar and wind microgrid, estimates its generation, identifies its operating condition and anomalies, and recommends how the available energy should be allocated, while being honest about the difference between measured values, simulated values and model predictions.

## 1.6 OBJECTIVES

•  To design and build a low-cost ESP8266 sensor node that measures solar voltage and current, wind voltage and current, turbine speed, temperature, humidity and light level from a hybrid solar and wind microgrid.

•  To develop a Flask REST backend that validates, deduplicates and stores the incoming telemetry in a SQLite database and exposes the data through a documented JSON API.

•  To engineer physically meaningful diagnostic features from the raw sensor vector and to train Random Forest models for generation prediction, operating-condition classification and anomaly detection.

•  To build an interactive web dashboard that shows live telemetry, historical trends, model outputs and an energy-allocation recommendation.

•  To establish a reproducible evaluation using a fixed-seed dataset and a chronological train, validation and test split, and to report the honest performance of each model.

•  To document the working principle, wiring, API contract and operating procedure so that the platform can be reproduced and extended.

## 1.7 SCOPE OF THE PROJECT

The project covers the sensor node, the backend, the machine learning pipeline, the dashboard and the evaluation. It is a laboratory prototype and uses low-voltage devices only. The relay output is held off by default and automatic load switching is deliberately not enabled, because safe switching requires a correctly rated relay or solid-state relay and a verified low-voltage bench test. At the time of writing the firmware has been compiled and verified, but the physical node has not been operated, so the machine learning models are trained and evaluated on simulated data.

## 1.8 ORGANISATION OF THE REPORT

Chapter 2 reviews the relevant literature and identifies the research gap. Chapter 3 describes the system architecture, the hardware, the firmware, the data contract, the database, the feature engineering and the machine learning models. Chapter 4 presents the dataset characteristics, the model results and the dashboard validation. Chapter 5 concludes the work and lists the future scope. The appendices contain the pin configuration, a sample API exchange and the installation and operating guide.

CHAPTER 2

# LITERATURE REVIEW

## 2.1 INTRODUCTION

This chapter reviews published work on IoT architectures for energy monitoring, solar and wind generation modelling, machine learning for generation prediction, anomaly detection and energy management. The review is used to position the present project and to identify the gap that it addresses.

## 2.2 IOT ARCHITECTURES FOR ENERGY MONITORING

Al-Fuqaha et al. [1] survey the enabling technologies, protocols and applications of the Internet of Things and describe the layered architecture that most monitoring systems follow: a sensing layer, a network layer and an application layer. Lazarescu [9] demonstrates that a low-power wireless sensor platform can collect environmental data over long periods with minimal maintenance. Gungor et al. [5] review communication technologies and standards for the smart grid and note that the choice of communication layer is driven by cost, range and data rate. These works establish that a microcontroller with integrated Wi-Fi and a lightweight HTTP protocol is sufficient for low-rate energy telemetry.

## 2.3 SOLAR AND WIND GENERATION MODELLING

Antonanzas et al. [2] review photovoltaic power forecasting and classify the methods into physical, statistical and machine learning families. They observe that the terminal voltage of a panel follows a saturating curve against irradiance and that cell temperature modifies this curve, which is the basis of the voltage-deficit diagnostic feature used in this project. For wind, the standard model relates the turbine speed to wind speed through a cut-in speed, a near-linear rise and a rated region, and the rectified voltage of a small generator is approximately proportional to the rotational speed above the rectifier drop. Voyant et al. [14] review machine learning methods for solar radiation forecasting and confirm that ensemble methods such as Random Forests are competitive and require little tuning.

## 2.4 MACHINE LEARNING FOR GENERATION PREDICTION

Breiman [3] introduced Random Forests, an ensemble of decision trees trained on bootstrap samples with random feature subsets, and showed that the method is accurate, robust to outliers and provides a natural measure of feature importance. Raza and Khosravi [12] review artificial-intelligence-based load and generation forecasting for smart grids and buildings and report that ensemble tree methods perform well on tabular sensor data. Kanchev et al. [8] present energy management and operational planning for a microgrid with a photovoltaic generator and show how a forecast of the available power improves the dispatch decision.

## 2.5 ANOMALY AND FAULT DETECTION

Chandola et al. [4] survey anomaly detection and distinguish point, contextual and collective anomalies. Sensor faults, stuck sensors, generation drops, generator mismatch and over-voltage spikes are point anomalies that appear as unusual combinations of features rather than as unusual single values. A supervised classifier can be trained to recognise these combinations when a labelled fault history is available, which is the approach taken here. Han et al. [6] describe the preprocessing, feature construction and evaluation methodology used for such classification tasks.

## 2.6 ENERGY MANAGEMENT AND LOAD ALLOCATION

Hatziargyriou et al. [7] introduce microgrids and describe their control and management requirements. Siano [13] reviews demand response and smart grids and highlights the role of real-time information in shifting flexible load to periods of surplus generation. Lund [10] argues that renewable energy strategies for sustainable development depend on the integration of variable sources with flexible demand and storage. These works motivate the allocation policy implemented in this project, in which available generation is assigned first to essential load, then to a reserve, then to deferrable load and finally to storage.

## 2.7 RESEARCH GAP

The reviewed literature provides strong evidence for each individual building block, but it rarely combines them into a single, open, low-cost platform for a hybrid solar and wind microgrid. Table 2.1 summarises the relevant studies and shows that most concentrate on a single source, on a single task or on expensive instrumentation. The present project addresses this gap by integrating a low-cost sensor node, a documented REST backend, a transparent feature-engineering step, three Random Forest models and an advisory energy-allocation policy in one reproducible system, with an explicit separation of measured, simulated and predicted values.

Table 2.1  Summary of related work and the research gap

| Study | Focus | Technique | Limitation |
| --- | --- | --- | --- |
| Al-Fuqaha et al. [1] | IoT architecture and protocols | Survey | No energy application |
| Lazarescu [9] | Low-power WSN monitoring | Embedded platform | Environmental data only |
| Antonanzas et al. [2] | PV power forecasting | Review of methods | Solar only, no node design |
| Voyant et al. [14] | Solar radiation forecasting | Machine learning review | No wind channel, no dashboard |
| Breiman [3] | Ensemble learning | Random Forest | Algorithm only |
| Raza and Khosravi [12] | Load and generation forecasting | AI review | No hardware integration |
| Chandola et al. [4] | Anomaly detection | Survey | Generic, not microgrid-specific |
| Kanchev et al. [8] | Microgrid energy management | Optimisation and control | Requires accurate forecasts |
| Siano [13] | Demand response | Review | No prediction models |
| This work | Hybrid solar and wind microgrid | IoT node + Random Forest + dashboard | Simulated evaluation, no field trial |

## 2.8 SUMMARY

The literature confirms the value of IoT sensing, of Random Forest models for prediction and of transparent energy management, but no reviewed work brings these together for a low-cost hybrid solar and wind microgrid with the same attention to honesty about measured, simulated and predicted data. The next chapter describes the system that was built to fill this gap.

CHAPTER 3

# SYSTEM DESIGN AND METHODOLOGY

## 3.1 INTRODUCTION

This chapter describes how the platform was designed and built. It covers the overall architecture, the hardware, the sensor node firmware, the data contract, the database, the feature engineering, the synthetic dataset, the machine learning models, the energy-allocation policy, the dashboard and the testing approach.

## 3.2 SYSTEM ARCHITECTURE

The system is organised as a sensing layer, a network and storage layer and an application layer, as shown in Figure 3.1. The sensor node reads the electrical, mechanical and environmental sensors, builds one JSON reading and posts it to the backend over the local Wi-Fi network. The backend validates the reading, stores it in a SQLite database, runs the three machine learning models and serves the dashboard. The dashboard runs entirely on the laptop that hosts the backend; the ESP8266 only publishes readings and never receives control commands.

Figure 3.1  System architecture of the IoT microgrid monitoring platform

## 3.3 HARDWARE DESIGN

The node is built around the ESP8266 NodeMCU, which provides Wi-Fi connectivity, one analogue input and sufficient digital pins for the sensors. The generators are a small 6 V solar panel and a small DC wind dynamo. Because the microgrid is a low-voltage laboratory prototype, the measured voltages are small and the currents are in the milliampere range. Table 3.1 lists the components and their roles.

Table 3.1  Hardware components and their roles

| Component | Specification | Role |
| --- | --- | --- |
| ESP8266 NodeMCU | ESP-12E, 80 MHz, 3.3 V logic | Microcontroller and Wi-Fi node |
| Solar panel | 6 V, 1.5 W | Solar generation source |
| DC wind dynamo | Small DC generator with propeller | Wind generation source |
| INA219 (x2) | I2C, addresses 0x40 and 0x41 | Bus voltage and current measurement |
| DHT11 | 3.3 V, digital | Temperature and humidity |
| KY-024 Hall module | Digital output | Turbine revolution counting |
| LDR module | Comparator, digital output | Bright and dark light detection |
| Relay module | Active-low, default OFF | Advisory load switching (disabled) |
| Resistors | 10 kohm pull-up, load resistors | DHT11 pull-up and test load |

## 3.4 SENSOR NODE FIRMWARE

The firmware is written for the NodeMCU 1.0 (ESP-12E) board and is designed to stay responsive: the main loop never blocks for more than a few milliseconds and uses non-blocking retries for the Wi-Fi link and for any sensor that failed at boot. Two INA219 modules share the I2C bus and are distinguished by their addresses. The Hall-effect module generates an interrupt on each magnet pass and the revolutions per minute are computed from the pulse count and the elapsed interval. The pin map is given in Table 3.2.

Table 3.2  ESP8266 pin map

| Signal | Pin | GPIO | Notes |
| --- | --- | --- | --- |
| I2C SDA | D2 | GPIO4 | Shared by both INA219 modules |
| I2C SCL | D1 | GPIO5 | Shared by both INA219 modules |
| DHT11 data | D5 | GPIO14 | 10 kohm pull-up to 3V3 |
| Hall digital out | D6 | GPIO12 | Interrupt, pulse counting |
| LDR digital out | D7 | GPIO13 | Bright and dark detection |
| Relay input | D0 | GPIO16 | Active-low, held OFF |
| On-board LED | D4 | GPIO2 | Heartbeat blink |

The firmware sends a field as null rather than guessing it. In particular, generated current cannot be measured when no load is connected, because the INA219 measures current through its shunt and an open circuit carries no current; in that case the current and power fields are sent as null and the dashboard shows that the current was not measured. The relay is driven to its off state during start-up and is never switched automatically.

## 3.5 DATA CONTRACT AND REST API

The node posts a JSON object to the ingest endpoint. The backend converts the firmware field names to internal column names, parses the timestamp, checks every numeric field against a plausible range and rejects a reading with a precise reason when it is not usable. Table 3.3 lists the payload fields and Table 3.4 lists the endpoints exposed by the backend.

Table 3.3  Ingest payload fields

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| device_id | string | No | Node identifier |
| timestamp | ISO-8601 | No | UTC sample time |
| solar_voltage | number | Yes | Panel bus voltage (V) |
| solar_current | number or null | No | Panel current (A); null when not measurable |
| solar_power | number or null | No | Panel power (W) |
| wind_voltage | number | Yes | Dynamo bus voltage (V) |
| wind_current | number or null | No | Dynamo current (A) |
| wind_power | number or null | No | Dynamo power (W) |
| turbine_rpm | number | Yes | Turbine speed (RPM) |
| temperature | number | No | Ambient temperature (degC) |
| humidity | number | No | Relative humidity (%) |
| light_status | string | No | bright, dim, dark or unknown |
| sensor_status | string | Yes | OK or a list of failures |
| device_ip | string | No | Node IP address |

Table 3.4  REST API endpoints

| Method | Endpoint | Purpose |
| --- | --- | --- |
| POST | /api/ingest | Validate, store and predict one reading |
| GET | /api/health | Service and model readiness |
| GET | /api/overview | Dashboard aggregate payload |
| GET | /api/readings/latest | Most recent reading and prediction |
| GET | /api/readings/history | Historical readings for charts |
| GET | /api/predictions/series | Predictions joined to readings |
| POST | /api/predict | Predict for a supplied or latest reading |
| GET | /api/model/status | Model metadata and test metrics |
| GET | /api/model/importance | Feature importances |
| GET/PUT | /api/settings | Runtime configuration |
| POST | /api/demo/seed | Insert simulated readings for demonstration |
| GET | /api/export/readings.csv | Download readings as CSV |
| POST | /api/maintenance/purge | Delete stored readings |

## 3.6 DATABASE DESIGN

The backend stores data in a SQLite database with four tables. Readings are deduplicated on the pair of device identifier and reading timestamp, so a node that reconnects and re-sends the same sample cannot pollute the history. The entity-relationship diagram is shown in Figure 3.2 and the tables are described in Table 3.5.

Figure 3.2  Entity-relationship diagram of the SQLite database

Table 3.5  Database tables and their purpose

| Table | Purpose | Key columns |
| --- | --- | --- |
| readings | One row per accepted sample | id, device_id, reading_timestamp, sensor values |
| devices | Latest state of each node | device_id, last_seen_unix, total_readings |
| predictions | Model output per reading | reading_id, predicted_power_w, predicted_condition |
| settings | Runtime configuration | key, value, updated_at_unix |

## 3.7 DATA PIPELINE AND FEATURE ENGINEERING

The platform has two pipelines that share the same feature engineering code so that training and live inference remain consistent. The online pipeline processes live readings and the offline pipeline generates the dataset, trains the models and produces the evaluation figures. Both pipelines are shown in Figure 3.3.

Figure 3.3  Online and offline data pipelines sharing one feature-engineering implementation

Three diagnostic features are computed from the raw sensor vector and fixed hardware constants. The solar voltage deficit compares the measured panel voltage with the clean voltage expected for the current irradiance and cell temperature, so partial shading and heavy soiling appear here first. The wind volts-per-RPM feature measures the effective generator constant, which falls when the rotor is fouled or the rectifier begins to fail. The lux-per-irradiance feature measures the agreement between the light sensor and the irradiance estimate, so a stuck or shaded light sensor becomes visible. Table 3.6 lists the features used by every model.

Table 3.6  Features used by the machine learning models

| Feature | Definition |
| --- | --- |
| hour | Hour of day from the reading timestamp |
| day_of_year | Day of year from the reading timestamp |
| solar_irradiance_wm2 | Irradiance inferred from the LDR |
| solar_voltage | Measured panel bus voltage (V) |
| wind_voltage | Measured dynamo bus voltage (V) |
| turbine_rpm | Measured turbine speed (RPM) |
| temperature_c | Ambient temperature (degC) |
| humidity_percent | Relative humidity (%) |
| light_level_lux | Light level (lux) |
| solar_v_deficit | Clean panel voltage minus measured voltage |
| wind_v_per_rpm | Dynamo voltage divided by RPM |
| lux_per_irradiance | Light level divided by irradiance |

## 3.8 DATASET GENERATION

Because the physical node could not be operated during the evaluation, a reproducible synthetic dataset was generated to develop and demonstrate the pipeline. The generator models a solar irradiance curve that depends on the time of day and on cloud cover, a temperature and humidity cycle, a wind-speed process with gusts, the panel voltage curve and the rectified dynamo voltage curve, and the non-fault loss mechanisms of soiling and becalming. It then injects five labelled faults: a sensor fault, a stuck sensor, a generation drop, a voltage-to-RPM mismatch and an over-voltage spike. The dataset contains 12,000 samples at a five-minute interval, spanning 01 January 2025 to 11 February 2025, and is fully reproducible for the fixed seed 42.

## 3.9 MACHINE LEARNING MODELS

Three supervised models are trained on the engineered features. The generation regressor predicts the total estimated power in watts. The operating-condition classifier predicts one of four labels: normal, reduced generation, night idle or fault. The anomaly classifier predicts whether the reading contains an injected sensor or equipment anomaly. All three are Random Forest models, which were chosen for their accuracy on tabular data, their robustness to noise and the interpretable feature importances they provide. The hyperparameters are listed in Table 3.7.

Table 3.7  Model hyperparameters

| Model | Estimators | Other settings | Target |
| --- | --- | --- | --- |
| Generation regressor | 220 | min_samples_leaf=2, max_features=0.8 | total_est_power_w |
| Condition classifier | 220 | balanced_subsample, min_samples_leaf=2 | operating_condition |
| Anomaly classifier | 220 | balanced_subsample, min_samples_leaf=2 | anomaly_label |

The dataset is divided chronologically into 70 % training, 15 % validation and 15 % test partitions. A random shuffle is deliberately avoided so that the model cannot see the future, which would inflate every reported score. The validation set is used for sanity checks and the test set is used exactly once, to report the final honest numbers. The fixed random state ensures that the whole experiment can be reproduced.

## 3.10 ENERGY ALLOCATION POLICY

The energy-allocation component is a transparent rule-based policy rather than a learned controller. It answers a single question: given the power that is being generated right now, how should it be split between essential load, a reserve, deferrable load and storage. Table 3.8 lists the tiers of the policy. The relay is never switched automatically, and the recommendation becomes advisory only whenever the node is offline or the generation cannot be measured.

Table 3.8  Energy-allocation tiers and default values

| Tier | Description | Default |
| --- | --- | --- |
| Essential load | Served first, unconditionally | 1.5 W |
| Reserve | Held back before discretionary load | 5.0 W |
| Deferrable load | Served only if surplus remains | 4.0 W |
| Storage | Receives any remaining surplus | Remainder |

## 3.11 DASHBOARD

The dashboard is a single-page web application served by the same Flask process. It uses a dark charcoal theme with an emerald-green accent and a local copy of the Chart.js library so that it works without an internet connection. It presents seven sections: an overview, live monitoring, AI analytics, energy optimization, historical data, model and system status, and settings. The settings page never reveals the device token or any stored secret in full.

## 3.12 SOFTWARE AND TOOLCHAIN

The software is written in Python and JavaScript. Table 3.9 lists the principal tools and their roles. The platform runs on Python 3.8 and is served by the built-in Flask development server for demonstration.

Table 3.9  Software, libraries and their roles

| Tool / Library | Version | Role |
| --- | --- | --- |
| Python | 3.8 | Backend and machine learning |
| Flask | 2.3.3 | REST API and dashboard server |
| pandas / numpy | 2.0.3 / 1.24.4 | Data processing |
| scikit-learn | 1.3.2 | Random Forest models |
| joblib | 1.4.2 | Model persistence |
| matplotlib / seaborn | 3.7.2 / 0.12.2 | Evaluation figures |
| SQLite | bundled | Persistent storage |
| Chart.js | 4.4.1 | Dashboard charts |
| Arduino ESP8266 core | 3.1.2 | Firmware compilation |

## 3.13 TESTING APPROACH

The backend is covered by an automated test suite that runs against a temporary database so that the development database is never modified. The suite exercises payload validation, the database layer, every API route, the dataset generator, the training pipeline and the inference service. One hundred and five tests are defined, and a training test runs a real, small fit to confirm that the whole pipeline works end to end.

## 3.14 SUMMARY

This chapter described the architecture, the hardware, the firmware, the data contract, the database, the feature engineering, the dataset, the machine learning models, the allocation policy, the dashboard and the testing approach. The next chapter presents the results obtained from this system.

CHAPTER 4

# RESULTS AND DISCUSSION

## 4.1 INTRODUCTION

This chapter presents and discusses the results. It begins with the characteristics of the dataset, then examines the sensor and generation relationships, then reports the performance of the three models on the held-out test set, then presents the energy-allocation results and the dashboard validation. It concludes with the limitations of the evaluation.

## 4.2 DATASET CHARACTERISTICS

The generated dataset contains 12,000 samples and 27 columns, with no null values and no duplicated timestamps. Table 4.1 summarises its composition. The anomaly types are distributed as shown in Table 4.2 and the operating conditions as shown in Figure 4.1; the dataset is imbalanced, with normal operation dominating and fault conditions being relatively rare, which is why the classifiers use balanced class weighting.

Table 4.1  Dataset composition

| Property | Value |
| --- | --- |
| Number of samples | 12,000 |
| Number of columns | 27 |
| Sample interval | 5 minutes |
| Time range | 01 Jan 2025 to 11 Feb 2025 |
| Null cells | 0 |
| Anomaly rate | 7.49 % |
| Random seed | 42 |
| Train / validation / test | 8400 / 1800 / 1800 |

Table 4.2  Anomaly-type distribution

| Anomaly type | Count |
| --- | --- |
| none | 11101 |
| generation_drop | 268 |
| sensor_fault | 241 |
| voltage_rpm_mismatch | 197 |
| overvoltage_spike | 124 |
| sensor_stuck | 69 |

Figure 4.1  Distribution of operating conditions in the dataset

## 4.3 SENSOR AND GENERATION ANALYSIS

Figure 4.2 shows the panel voltage and the solar power over time. The voltage rises quickly with irradiance and then saturates, which is the expected behaviour of a small panel. Figure 4.3 shows the dynamo voltage and the turbine speed over time and confirms that the rectified voltage tracks the speed above the rectifier drop. Figure 4.4 shows that the light level and the solar generation are strongly related, which supports the use of the light sensor as a proxy for irradiance. Figure 4.5 shows the relationship between the turbine speed and the wind generation. Figure 4.6 shows the relative contribution of the solar and wind channels to the total generation and illustrates the complementary nature of the two sources.

Figure 4.2  Solar voltage and solar power over time

Figure 4.3  Wind voltage and turbine RPM over time

Figure 4.4  Light level versus solar generation

Figure 4.5  Turbine RPM versus wind generation

Figure 4.6  Relative contribution of solar and wind generation

## 4.4 GENERATION PREDICTION RESULTS

The generation regressor was evaluated on the 1,800 held-out test samples. Table 4.3 reports the metrics on both the validation and the test partitions. The mean absolute error on the test set is 0.0147 W and the coefficient of determination is 0.9968, which means that almost all of the variation in the generation is explained by the sensor features. Figure 4.7 shows the predicted power against the actual power and Figure 4.8 shows the regression evaluation. Figure 4.9 shows the generation trend and forecast over the test period. Figure 4.10 compares the model scores for a compact summary.

Table 4.3  Generation regressor performance

| Metric | Validation | Test |
| --- | --- | --- |
| Mean absolute error (W) | 0.0202 | 0.0147 |
| Root mean square error (W) | 0.0399 | 0.0251 |
| Coefficient of determination | 0.9926 | 0.9968 |
| Maximum absolute error (W) | 0.8459 | 0.2018 |

Figure 4.7  Actual versus predicted power on the held-out test set

Figure 4.8  Regression evaluation and residuals

Figure 4.9  Generation trend and forecast

Figure 4.10  Comparison of model scores

## 4.5 OPERATING-CONDITION CLASSIFICATION RESULTS

The operating-condition classifier achieved an accuracy of 0.8528 and a macro F1 score of 0.8365 on the held-out test set. Table 4.4 reports the per-class precision, recall and F1 score, and Figure 4.11 shows the confusion matrix. The night-idle and fault classes are detected reliably, while the reduced-generation class is the most difficult, because a moderate reduction in output lies close to normal operation and is sometimes misclassified as normal. This is an acceptable trade-off for a monitoring system, in which a missed early degradation is less dangerous than frequent false alarms.

Table 4.4  Operating-condition classification report (held-out test set)

| Class | Precision | Recall | F1 score | Support |
| --- | --- | --- | --- | --- |
| fault | 0.953 | 0.795 | 0.867 | 127 |
| night_idle | 0.976 | 1.000 | 0.988 | 82 |
| normal | 0.878 | 0.933 | 0.904 | 1261 |
| reduced_generation | 0.652 | 0.533 | 0.587 | 330 |
| Macro average | 0.865 | 0.815 | 0.837 | 1800 |

Figure 4.11  Confusion matrix for the operating-condition classifier

## 4.6 ANOMALY DETECTION RESULTS

The anomaly detector achieved an accuracy of 0.9861 on the held-out test set, with a precision of 0.9636 and a recall of 0.8346. Table 4.5 reports the metrics and Figure 4.12 shows the confusion matrix. The detector is precise, which means that it rarely raises a false alarm, and it recovers a large fraction of the injected anomalies. The false negatives are predominantly the mild generation drops that are hard to separate from legitimate soiling. Figure 4.13 contrasts the normal and anomalous patterns.

Table 4.5  Anomaly detector performance on the held-out test set

| Metric | Value |
| --- | --- |
| Accuracy | 0.9861 |
| Precision | 0.9636 |
| Recall | 0.8346 |
| F1 score | 0.8945 |
| Specificity | 0.9976 |
| True positives / false negatives | 106 / 21 |
| False positives / true negatives | 4 / 1669 |

Figure 4.12  Confusion matrix for the anomaly detector

Figure 4.13  Normal versus anomalous sensor patterns

## 4.7 FEATURE IMPORTANCE

Figure 4.14 shows the feature importances of the generation regressor and Table 4.6 lists them. The two measured voltages dominate, which is expected because they are directly related to the generated power. The turbine speed and the irradiance estimate are the next most important, and the engineered diagnostic features contribute the remainder. The ordering confirms that the model is learning physically meaningful relationships rather than memorising the data.

Table 4.6  Feature importance of the generation regressor

| Feature | Importance |
| --- | --- |
| solar_voltage | 0.4122 |
| wind_voltage | 0.3773 |
| turbine_rpm | 0.0734 |
| solar_irradiance_wm2 | 0.0385 |
| light_level_lux | 0.0372 |
| lux_per_irradiance | 0.0324 |
| wind_v_per_rpm | 0.0247 |
| solar_v_deficit | 0.0028 |

Figure 4.14  Feature importance of the generation regressor

## 4.8 ENERGY OPTIMIZATION RESULTS

The allocation policy was applied to a set of representative generation levels. Table 4.7 shows four scenarios. When the generation is zero, the status is unknown and a recommendation is produced without switching the relay. When the generation is below the demand, the status is deficit and the policy advises shedding the deferrable load before the essential load. When the generation just covers the demand, the status is balanced. When the generation exceeds the demand and the reserve, the surplus is assigned to storage and the status is surplus.

Table 4.7  Energy-allocation scenarios

| Scenario | Generation | Status | Essential | Deferrable | Storage |
| --- | --- | --- | --- | --- | --- |
| A | 0.0 W | unknown | 0.0 | 0.0 | 0.0 |
| B | 1.0 W | deficit | 1.0 | 0.0 | 0.0 |
| C | 5.5 W | balanced | 1.5 | 4.0 | 0.0 |
| D | 8.0 W | surplus | 1.5 | 4.0 | 2.5 |

The essential-load tier is always served first, the reserve is held back before any discretionary load, and the deferrable load is served only while a surplus remains. This policy is transparent and easy to explain to an operator, which was the intended design goal.

## 4.9 API AND DASHBOARD VALIDATION

The backend was started and exercised with real HTTP requests. The health endpoint reported the service as healthy with the models loaded, the overview and history endpoints returned valid payloads, the dashboard page was served correctly, the evaluation figures were served as static files and an unknown figure correctly returned a not-found response. A telemetry tick inserted a simulated reading and produced a prediction, which confirms that the ingestion and inference path works end to end.

## 4.10 LIMITATIONS OF THE EVALUATION

The reported metrics characterise the software pipeline on simulated data and are not a claim of field accuracy. The synthetic faults are cleanly separable, so the measured F1 scores overstate what a real, noisy sensor stream would produce. The estimated currents come from the same simulation model as the target, so the regression task is easier than a real one. The physical node has not been operated, so no field measurement of the sensors or of the models has been made. Finally, no battery state of charge, cloud forecast or tariff signal is modelled, so the optimiser remains a rule-based demonstration.

## 4.11 SUMMARY

The results show that the pipeline predicts generation with a very small error, classifies the operating condition with good accuracy and detects anomalies with high precision, all on a held-out chronological test set. The limitations of simulating the data are stated explicitly, and the next chapter concludes the work and describes how these limitations can be removed.

CHAPTER 5

# CONCLUSION AND FUTURE SCOPE

## 5.1 CONCLUSION

This project designed, built and evaluated a low-cost IoT platform for the predictive monitoring and energy optimization of a hybrid solar and wind microgrid. The platform integrates an ESP8266 sensor node, a Flask REST backend with SQLite storage, a transparent feature-engineering step, three Random Forest models and a dark-themed web dashboard. Measured values, simulated values and model predictions are kept separate throughout so that no estimated number is ever presented as a measurement.

On a reproducible simulated dataset of 12,000 samples with a strictly chronological split, the generation regressor achieved a mean absolute error of 0.0147 W and a coefficient of determination of 0.9968, the operating-condition classifier achieved an accuracy of 0.8528 with a macro F1 score of 0.8365, and the anomaly detector achieved an accuracy of 0.9861 with a precision of 0.9636 and a recall of 0.8346. The energy allocation policy produced transparent and explainable recommendations for essential load, reserve, deferrable load and storage, while keeping the relay off by default for safety.

The objectives set out in Chapter 1 were met: the node, the backend, the models and the dashboard were all implemented and verified as software, the evaluation was made reproducible, and the system was documented so that it can be reproduced and extended. The principal limitation is that the physical node was not operated during the evaluation, so the reported metrics reflect the software pipeline rather than proven field performance.

## 5.2 FUTURE SCOPE

•  Deploy the node on the physical bench, replace the synthetic dataset with a real sensor log and retrain the models to obtain genuine field accuracy figures.

•  Add an accurate pyranometer or calibrate the light-sensor curve against a reference so that the irradiance estimate improves.

•  Introduce a battery model with a state-of-charge estimate and a simple charge and discharge controller.

•  Integrate a short-horizon weather forecast so that the allocation policy can plan ahead instead of reacting to the current reading.

•  Enable the relay for automatic load switching only after a verified low-voltage bench test with a correctly rated relay or solid-state relay.

•  Add multi-node support so that several microgrid nodes can be monitored from a single dashboard.

•  Add authentication and transport security so that the platform can be deployed beyond a trusted local network.

•  Package the platform as a container with a production WSGI server for long-term unattended operation.

## 5.3 SUSTAINABLE DEVELOPMENT GOALS

The project supports SDG 7 (Affordable and Clean Energy) by improving the usable output of renewable generation, SDG 9 (Industry, Innovation and Infrastructure) through the use of low-cost IoT and machine learning, and SDG 13 (Climate Action) by encouraging the efficient use of clean energy.

# APPENDIX 1 - PIN CONFIGURATION AND WIRING

The node uses the default I2C pins of the ESP8266, so both INA219 modules share the same SDA and SCL lines and are distinguished by their addresses. All modules and both generators must share a common ground.

Table A1.1  Connection summary

| Module | Module pin | ESP8266 pin | Remarks |
| --- | --- | --- | --- |
| INA219 solar | VCC / GND / SDA / SCL | 3V3 / GND / D2 / D1 | Address 0x40 |
| INA219 wind | VCC / GND / SDA / SCL | 3V3 / GND / D2 / D1 | Address 0x41 (A0 bridged) |
| DHT11 | VCC / GND / DATA | 3V3 / GND / D5 | 10 kohm pull-up on DATA |
| KY-024 | VCC / GND / DO | 3V3 / GND / D6 | One pulse per magnet pass |
| LDR module | VCC / GND / DO | 3V3 / GND / D7 | Set active-low in firmware |
| Relay | VCC / GND / IN | VU / GND / D0 | Active-low, default OFF |

# APPENDIX 2 - SAMPLE API EXCHANGE

The following example shows a single reading posted by the node and the response returned by the backend. The current and power fields are null because no load is connected.

Request  POST /api/ingest

Content-Type: application/json

X-Device-Token: <device-token>

{

"device_id": "ESP8266-GRID-01",

"timestamp": "2025-01-01T12:00:00+00:00",

"solar_voltage": 5.42, "solar_current": null, "solar_power": null,

"wind_voltage": 2.81, "wind_current": null, "wind_power": null,

"turbine_rpm": 640.0, "temperature": 28.5, "humidity": 61.0,

"light_status": "bright", "sensor_status": "OK",

"device_ip": "192.168.43.37"

}

Response  201 Created

{

"status": "ok",

"data": {

"accepted": true, "duplicate": false, "reading_id": 1,

"prediction": {

"predicted_power_w": 0.3917,

"predicted_condition": "normal",

"condition_confidence": 0.87,

"anomaly_probability": 0.02, "anomaly_flag": 0,

"model_version": "1.0.0"

}

},

"server_time": "2025-01-01T12:00:01+00:00"

}

# APPENDIX 3 - INSTALLATION AND OPERATING GUIDE

The project is distributed as a single archive. The following steps describe how to run the backend and the dashboard on a Windows machine.

Step 1: install Python 3.8 and the required packages with pip install -r requirements.txt, or run install.bat.

Step 2: generate the dataset and train the models by running python -m ml.generate_dataset followed by python -m ml.train, or run the helper script scripts/run_pipeline.py.

Step 3: start the backend with run.bat or with python -m backend.app. The dashboard is then available at the local host address on port 5000.

Step 4: copy firmware/microgrid_node/credentials.example.h to credentials.h, enter the Wi-Fi name, the Wi-Fi password and the backend address, and compile and upload the firmware with the Arduino tools.

Step 5: open the dashboard, confirm that the device appears as online and that readings are arriving, and use the settings page to adjust the sampling interval and the anomaly threshold. A demonstration feed can be started from the settings page when no hardware is connected.

# REFERENCES

Al-Fuqaha, A., Guizani, M., Mohammadi, M., Aledhari, M., & Ayyash, M. (2015). Internet of Things: A survey on enabling technologies, protocols, and applications. IEEE Communications Surveys & Tutorials, 17(4), 2347-2376. https://doi.org/10.1109/COMST.2015.2444095

Antonanzas, J., Osorio, N., Escobar, R., Urraca, R., Martinez-de-Pison, F. J., & Antonanzas-Torres, F. (2016). Review of photovoltaic power forecasting. Solar Energy, 136, 78-111. https://doi.org/10.1016/j.solener.2016.06.069

Breiman, L. (2001). Random forests. Machine Learning, 45(1), 5-32. https://doi.org/10.1023/A:1010933404324

Chandola, V., Banerjee, A., & Kumar, V. (2009). Anomaly detection: A survey. ACM Computing Surveys, 41(3), 1-58. https://doi.org/10.1145/1541880.1541882

Gungor, V. C., Sahin, D., Kocak, T., Ergut, S., Buccella, C., Cecati, C., & Hancke, G. P. (2011). Smart grid technologies: Communication technologies and standards. IEEE Transactions on Industrial Informatics, 7(4), 529-539. https://doi.org/10.1109/TII.2011.2166794

Han, J., Pei, J., & Tong, H. (2022). Data Mining: Concepts and Techniques (4th ed.). Morgan Kaufmann.

Hatziargyriou, N., Asano, H., Iravani, R., & Marnay, C. (2007). Microgrids. IEEE Power and Energy Magazine, 5(4), 78-94. https://doi.org/10.1109/MPAE.2007.376583

Kanchev, H., Lu, D., Colas, F., Lazarov, V., & Francois, B. (2011). Energy management and operational planning of a microgrid with a PV-based active generator for smart grid applications. IEEE Transactions on Industrial Electronics, 58(10), 4583-4592. https://doi.org/10.1109/TIE.2011.2123853

Lazarescu, M. T. (2013). Design of a WSN platform for long-term environmental monitoring for IoT applications. IEEE Journal on Emerging and Selected Topics in Circuits and Systems, 3(1), 45-54. https://doi.org/10.1109/JETCAS.2013.2243032

Lund, H. (2007). Renewable energy strategies for sustainable development. Energy, 32(6), 912-919. https://doi.org/10.1016/j.energy.2006.10.017

Pedregosa, F., Varoquaux, G., Gramfort, A., Michel, V., Thirion, B., Grisel, O., Blondel, M., Prettenhofer, P., Weiss, R., Dubourg, V., Vanderplas, J., Passos, A., Cournapeau, D., Brucher, M., Perrot, M., & Duchesnay, E. (2011). Scikit-learn: Machine learning in Python. Journal of Machine Learning Research, 12, 2825-2830.

Raza, M. Q., & Khosravi, A. (2015). A review on artificial intelligence based load demand forecasting techniques for smart grid and buildings. Renewable and Sustainable Energy Reviews, 50, 1352-1372. https://doi.org/10.1016/j.rser.2015.04.065

Siano, P. (2014). Demand response and smart grids - A survey. Renewable and Sustainable Energy Reviews, 30, 461-478. https://doi.org/10.1016/j.rser.2013.10.022

Voyant, C., Notton, G., Kalogirou, S., Nivet, M.-L., Paoli, C., Motte, F., & Fouilloy, A. (2017). Machine learning methods for solar radiation forecasting: A review. Renewable Energy, 105, 569-582. https://doi.org/10.1016/j.renene.2016.12.095